Hot work permit

Permit no. __________________________________   Date __________________________________

Location __________________________________   Time permit starts __________________________________

Time permit ends __________________________________   Hot work to be done __________________________________

Person doing the work __________________________________   Fire watch __________________________________

Authorized by ______________________________________________________________________

Before the work starts

OSHA 1910.252 asks the person who authorizes cutting and welding to inspect the area and set the precautions, preferably in a written permit. Construction work has its own rules in 29 CFR 1926 Subpart J. Check the current text and your own program.

| ☐ Area inspected by the person who authorizes the work | ☐ No explosive atmosphere, and no work in an area where one could form |
| --- | --- |
| ☐ Combustibles moved at least 35 feet (10.7 m) from the work, or shielded with flameproof covers or guards | ☐ Used containers cleaned and vented before any heat is applied |
| ☐ Floor swept clean for 35 feet, and wet or covered if it is combustible | ☐ Sprinklers in service, if the building has them |
| ☐ Floor and wall openings, cracks and open doorways covered so sparks cannot drop or travel | ☐ Fire extinguisher or hose ready for instant use |
| ☐ Ducts and conveyors that could carry sparks protected or shut down | ☐ Fire watch assigned, trained and holding a fire extinguisher |
| ☐ Combustible walls and partitions near the work shielded | ☐ Fire alarm location known to the welder and the fire watch |

Special conditions

Anything specific to this job.

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

After the work

The fire watch stays for at least a half hour after welding or cutting stops, to catch smoldering fires.

| Work finished (time) | Fire watch until (time) | Area checked by | Permit closed (initials) |
| --- | --- | --- | --- |

A starting point. Adapt it to your equipment, your procedures and the rules that apply to you. Free template from portalbaseai.com