Telehandler checklist

Date __________________________________   Time __________________________________

Site or location __________________________________   Operator name __________________________________

Machine make, model and unit number __________________________________   Hour meter reading __________________________________

Before you start (engine off)

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Operator's manual and the load chart are in the cab and readable | ☐ | ☐ | ☐ |
| Cab frame, overhead guard and glass are intact | ☐ | ☐ | ☐ |
| Seat, seat belt and any restraint are in good condition | ☐ | ☐ | ☐ |
| Boom sections, wear pads and chains show no cracks, bends or wear | ☐ | ☐ | ☐ |
| Forks or attachment are undamaged, and the carriage latch locks fully | ☐ | ☐ | ☐ |
| Stabilizers or outriggers extend, retract and lock, where fitted | ☐ | ☐ | ☐ |
| Hydraulic hoses, cylinders and fittings are free of leaks | ☐ | ☐ | ☐ |
| Tires and wheels are in good condition, and lug nuts are tight | ☐ | ☐ | ☐ |
| Engine oil, coolant, hydraulic fluid and fuel levels are correct | ☐ | ☐ | ☐ |
| Mirrors, lights and warning labels are clean and undamaged | ☐ | ☐ | ☐ |

After you start (engine on)

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Gauges and warning lights work, and none stay on | ☐ | ☐ | ☐ |
| Horn, back-up alarm and lights work | ☐ | ☐ | ☐ |
| Steering modes work: front, crab and four-wheel | ☐ | ☐ | ☐ |
| Service brake stops the machine, and the parking brake holds | ☐ | ☐ | ☐ |
| Boom lifts, extends, lowers and retracts smoothly, and holds when released | ☐ | ☐ | ☐ |
| Attachment tilt and any side-shift work smoothly | ☐ | ☐ | ☐ |
| Frame leveling works, where fitted | ☐ | ☐ | ☐ |
| Load moment indicator or load-limit warning works, where fitted | ☐ | ☐ | ☐ |
| No unusual noises, smells or vibration | ☐ | ☐ | ☐ |
| Fire extinguisher is in place, if your site requires one | ☐ | ☐ | ☐ |

Load and route

| Item | Pass | Fail | N/A |
| --- | --- | --- | --- |
| Load weight and its center are known and within the load chart for this reach and height | ☐ | ☐ | ☐ |
| Load is secure on the forks or attachment | ☐ | ☐ | ☐ |
| Ground is firm and level for the lift, with no soft spots or slopes | ☐ | ☐ | ☐ |
| Travel route is clear of people, power lines and overhead hazards | ☐ | ☐ | ☐ |
| Operator is trained on this machine and reads the load chart | ☐ | ☐ | ☐ |

Defects found and action taken

____________________________________________________________________________________________________

____________________________________________________________________________________________________

____________________________________________________________________________________________________

Taken out of service?  ☐ Yes   ☐ No        Reported to ______________________

Inspector signature __________________________________   Date __________________________________

Supervisor review __________________________________   Date __________________________________

A starting point. Adapt it to your equipment, your procedures and the rules that apply to you. Free template from portalbaseai.com